!..............................................................................................

!..............................................................................................

!REMOTE

!T2 z-side

!..............................................................................................

!..............................................................................................

!

!+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

!+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

!0 - variables

!

!FRONT DOOR a and z

!frontdoor-1-vrf-name

<<<<fd1-name>>>> fvrf-untrusted-wan-isp-1

!

!layer3-interface_phys-or-subint for fd1 on a side

<<<<L3-fd1-interface-a>>>>

!

!layer3-interface_phys-or-subint for fd1 on z side

<<<<L3-fd1-interface-z>>>>

!

!gateways

!a-side

!  <<<<a.a.a.gw>>>> 1.1.1.1

!z-side

!  <<<<z.z.z.gw>>>> 2.2.2.1

!

!

!ADDRESSES a and z

!a-side public outside interface ip address (in fd1 on a-side)

<<<<a.a.a.#>>>> 1.1.1.2

!a-side public outside interface ip address SUBNET

<<<<a.a.a.a_SUBNET>>>> 1.1.1.0

!a-side subnet mask for public outside interface ip address

<<<<a.a.a.#_CIDRmask>>>> 255.255.255.0

!a-side subnet mask for public outside interface ip address

<<<<a.a.a.#_WCmask>>>> 0.0.0.255

!

!z-side public outside interface ip address (in fd1 on z-side)

<<<<z.z.z.#>>>> 2.2.2.2

!z-side public outside interface ip address SUBNET

<<<<z.z.z.z_SUBNET>>>> 2.2.2.0

!z-side subnet mask for public outside interface ip address

<<<<z.z.z.#_CIDRmask>>>> 255.255.255.0

!z-side subnet mask for public outside interface ip address

<<<<z.z.z.#_WCmask>>>> 0.0.0.255

!

!

!1 - authentication

!

!aaa new-model

!aaa authentication login default local

aaa authorization network default local

!

!_______________________________________________________________________________________________

!2 - front-door-vrf

!

! protections

no ip http server

no ip http secure-server

!

vrf definition <<<<fd1-name>>>>

description FVRF1-UNTRUSTED

rd 65000:1

address-family ipv4

exit-address-family

!

!_______________________________________________________________________________________________

!3 - IKE2 policies

!

crypto logging session

!

no crypto ikev2 policy default

no crypto ikev2 proposal default

!

crypto ikev2 authorization policy default

route set interface

!

crypto ikev2 proposal IKEV2-PROPOSAL-1

encryption aes-cbc-256

integrity sha512

group 21

!_______________________________________________________________________________________________

!4 - IPSEC policies

!

crypto ipsec transform-set SHA384 esp-aes 256 esp-sha384-hmac

mode tunnel

!

!_______________________________________________________________________________________________

!5 - TUNNELS // IKE2

!

crypto ikev2 policy IKEV2-POLICY-1

match fvrf <<<<fd1-name>>>>

proposal IKEV2-PROPOSAL-1

!

crypto ikev2 keyring UNTRUSTED-WAN-KEYRING-1

!

peer PEER-A-WAN-1

description PEER-A-WAN-1

address <<<<a.a.a.#>>>>

pre-shared-key FVRF-IKEV2-PSK_KEYNAME

!

peer PEER-Z-WAN-1

description PEER-Z-WAN-1

address <<<<z.z.z.#>>>>

pre-shared-key FVRF-IKEV2-PSK_KEYNAME

!

crypto ikev2 profile FVRF-IKEV2-PROFILE-1-Z

description FVRF-IKEV2-PROFILE-1

match fvrf <<<<fd1-name>>>>

match identity remote address <<<<a.a.a.#>>>>

authentication remote pre-share

authentication local pre-share

keyring local UNTRUSTED-WAN-KEYRING-1

dpd 30 3 on-demand

aaa authorization group psk list default default

!

crypto ipsec profile INSIDE-IKEV2-PROFILE-TU999

description INSIDE-IKEV2-PROFILE-TU999

set transform-set SHA384

set pfs group21

set ikev2-profile FVRF-IKEV2-PROFILE-1

!

!_______________________________________________________________________________________________

!6 - loopbacks

interface lo999

description V-lo999-TU999-Z-SOURCE-172-30-99-2

ip address 172.30.99.2 255.255.255.255

!

!_______________________________________________________________________________________________

!7 - WAN-ISP-1 L3 interface

interface <<<<L3-fd1-interface-z>>>>

description PHYSICAL-UNTRUSTED-WAN-ISP-1-CONNECTION

vrf forwarding <<<<fd1-name>>>>

ip address <<<<z.z.z.#>>>> <<<<z.z.z.#_CIDRmask>>>>

ip access-group WAN-1-INGRESS-COMBINED in

ip access-group WAN-1-EGRESS-COMBINED out

no ip redirects

no ip unreachables

no ip mask-reply

no ip proxy-arp

cdp enable

no shut

!

!_______________________________________________________________________________________________

!8 - INSIDE TUNNEL // GRE protection

interface tu999

description V-GRE-TU999-VIA-untrusted-wan-isp1

ip unnumbered lo999

ip access-group to_Z-SIDE_TU999_IN in

ip access-group from_Z-SIDE_TU999_OUT out

no ip redirects

no ip unreachables

no ip proxy-arp

load-interval 30

tunnel source <<<<L3-fd1-interface-z>>>>

tunnel mode ipsec ipv4

tunnel destination <<<<a.a.a.#>>>>

tunnel vrf <<<<fd1-name>>>>

tunnel protection ipsec profile INSIDE-IKEV2-PROFILE-TU999

!_______________________________________________________________________________________________

!9 - Routing

!

ip route 172.30.99.0 255.255.255.0 tu999 name TU999-172-30-99-0

ip route 41.0.0.0 255.255.255.0 tu999 name A-41-NET

!_______________________________________________________________________________________________

!10 - PHYS WAN combined-ACL INGRESS (L3 PROTECTION) / WAN ACL EGRESS (VIEWING)

!

no ip access-list extended WAN-1-INGRESS-COMBINED

ip access-list extended WAN-1-INGRESS-COMBINED

deny ip any any option any-options log-input

permit tcp any any established

permit udp any eq bootps any eq bootpc

!

remark A-SIDE

permit icmp host <<<<a.a.a.gw>>>> any

permit icmp host <<<<a.a.a.#>>>> any

permit udp host <<<<a.a.a.#>>>> any eq isakmp

permit udp host <<<<a.a.a.#>>>> any eq non500-isakmp

permit esp host <<<<a.a.a.#>>>> any

!

remark Z-SIDE

permit icmp host <<<<z.z.z.gw>>>> any

permit icmp host <<<<z.z.z.#>>>> any

permit udp host <<<<z.z.z.#>>>> any eq isakmp

permit udp host <<<<z.z.z.#>>>> any eq non500-isakmp

permit esp host <<<<z.z.z.#>>>> any

!

deny udp any any eq isakmp log-input

deny udp any any eq non500-isakmp log-input

deny esp any any log-input

!

remark block bogons

!

remark explicit deny

deny ip any any log-input

!

!~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

!

no ip access-list extended WAN-1-EGRESS-COMBINED

ip access-list extended WAN-1-EGRESS-COMBINED

permit ip <<<<a.a.a.a_SUBNET>>>> <<<<a.a.a.#_WCmask>>>> <<<<z.z.z.z_SUBNET>>>> <<<<z.z.z.#_WCmask>>>>

permit ip <<<<z.z.z.z_SUBNET>>>> <<<<z.z.z.#_WCmask>>>> <<<<a.a.a.a_SUBNET>>>> <<<<a.a.a.#_WCmask>>>>

permit icmp <<<<a.a.a.a_SUBNET>>>> <<<<a.a.a.#_WCmask>>>> <<<<z.z.z.z_SUBNET>>>> <<<<z.z.z.#_WCmask>>>>

permit icmp <<<<z.z.z.z_SUBNET>>>> <<<<z.z.z.#_WCmask>>>> <<<<a.a.a.a_SUBNET>>>> <<<<a.a.a.#_WCmask>>>>

!

remark A-SIDE

permit icmp host <<<<a.a.a.#>>>> any log-input

permit udp host <<<<a.a.a.#>>>> any eq isakmp log-input

permit udp host <<<<a.a.a.#>>>> any eq non500-isakmp log-input

permit esp host <<<<a.a.a.#>>>> any log-input

!

remark Z-SIDE

permit icmp host <<<<z.z.z.#>>>> any log-input

permit udp host <<<<z.z.z.#>>>> any eq isakmp log-input

permit udp host <<<<z.z.z.#>>>> any eq non500-isakmp log-input

permit esp host <<<<z.z.z.#>>>> any log-input

!

!_______________________________________________________________________________________________

!11 - INSIDE TUNNEL ACL IN   >>>  to-Z_IN

no ip access-list extended to_Z-SIDE_TU999_IN

ip access-list extended to_Z-SIDE_TU999_IN

permit ip any any

!

!_______________________________________________________________________________________________

!12 - INSIDE TUNNEL ACL OUT   >>>   from-Z_OUT

no ip access-list extended from_Z-SIDE_TU999_OUT

ip access-list extended from_Z-SIDE_TU999_OUT

permit ip any any

!

!+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

!+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

!

! special vrf aliases

!

alias exec svb show vrf brief

alias exec sivi show ip vrf interfaces

!

alias exec pfvrf ping vrf <<<<fd1-name>>>>

alias exec safvrf show arp vrf <<<<fd1-name>>>>

alias exec sirfvrf show ip route vrf <<<<fd1-name>>>>

!

! special crypto ***SHOW-CLEAR*** aliases

!

alias exec v2sa show crypto ikev2 sa

alias exec secsa show crypto ipsec sa

alias exec clearv2 clear crypto ikev2 sa

alias exec clearsec clear crypto ipsec sa

!

! special crypto ***DEBUG*** aliases

!

alias exec debv2 debug crypto ikev2 packet

alias exec debv2all debug crypto ikev2 internal

alias exec debsecerror debug crypto ipsec error

alias exec debsecstate debug crypto ipsec states

!

alias exec wha sh run | i alias

alias exec siib show ip int brie

alias exec srint show run int

alias exec sir show ip route

alias exec sri sho run | i

alias exec srs sho run | sec

alias exec sid show int des

alias exec sis show int status

alias exec sit show int tru

alias exec sirs show ip route static

alias exec sidiu show int des | i up

alias exec caps sh crypto ipsec sa | i caps

alias exec sidp show int des | i P-

alias exec sidv show int des | i V-

alias exec ses show etherchannel sum

alias exec srin show run int

alias exec srsc show run | sec crypto

alias exec crs copy ru st

alias exec crf copy run bootflash:cfg-manual-backup

alias exec scn show cdp nei

alias exec sln show lldp nei

alias exec slndi show lldp nei det | i

alias exec scndi show cdp nei det | i